Helios_Artemis Documentation

Technical Documentation — Helios-Artemis MPPT Simulator

Hussain Touhid Siddiquee  |  Dept. EEE, Leading University Sylhet  |  Helios-Artemis Research Project

# 1. Purpose

CLEAN_AND_BUILD_v3.m is a MATLAB build automation script for the Helios-Artemis MPPT Simulator. It performs a complete teardown of any stale S-function source files and Simulink model, regenerates all six Level-2 S-function .m files programmatically, constructs the Simulink block diagram (HA_MPPT_v3.slx) from scratch using the Simulink API, and saves the model ready for simulation. The script is the single entry point that ensures a reproducible, clean simulation environment.

# 2. Quick-Start Run Sequence

Paste all four lines into the MATLAB Command Window in order:

clear log_ghi log_ppv log_pmpp log_eta log_soc log_vbat log_tj

clear mex

CLEAN_AND_BUILD_v3

sim('HA_MPPT_v3')

NOTE  Do NOT use 'clear all' inside or before the script — it will kill the script mid-execution. The 'clear mex' forces recompilation of S-functions and is required after any S-function source edit.

# 3. Script Architecture — 4 Steps

| Step | Label | Description |
| --- | --- | --- |
| 1 | Clean Slate | Closes HA_MPPT_v3 if loaded, deletes all 6 S-function .m files and post_v3.m from the working directory. |
| 2 | Write S-functions | Writes all 6 S-function source files via fopen/fprintf/fclose blocks, each implementing a discrete-time S-function interface. |
| 3 | Build Simulink Model | Creates HA_MPPT_v3 with new_system, configures solver, adds all blocks and subsystems, wires all signals, saves the .slx file. |
| 4 | Done | Calls addpath(pwd) and rehash to ensure all S-functions are on the path. Prints run instructions. |

# 4. S-Function Reference

All S-functions are discrete-time (Ts = 0.1 s) implemented via the legacy switch-flag interface. Each is fully regenerated on every script run.

## 4.1  ha_irr_v3.m — Markov Irradiance Model

Simulates stochastic global horizontal irradiance (GHI) for a Sylhet July day using a 3-state Markov chain combined with an Ornstein-Uhlenbeck (OU) flicker process.

| Parameter | Value / Description |
| --- | --- |
| States | 3 discrete: x(1)=cloud_state, x(2)=cloud_timer, x(3)=G_fast |
| Outputs | 1: GHI (W m⁻²) |
| Markov states | 0=overcast (cf=0.15), 1=partly cloudy (cf=0.55), 2=clear (cf=0.90) |
| Transition interval | 15 s — cloud state checked every 15 simulation seconds |
| OU flicker | tau=1.0 s, sigma_frac=0.25 (25% of Gcf), clipped to ±40% of clear-sky |
| Solar model | Gclear = 800 × aerosol × sin(π × fraction_of_day), aerosol = 0.93, peak ≈ 744 W m⁻² |
| Sunrise / Sunset | 05:30 / 18:30 local (Sylhet July) |

## 4.2  ha_lstm_v3.m — NARX Irradiance Predictor

Implements a 4-unit NARX (Nonlinear AutoRegressive with eXogenous inputs) network with tanh activation to predict irradiance 30 s ahead, with a stochastic UART hold to simulate embedded MCU communication latency.

| Parameter | Value / Description |
| --- | --- |
| States | 26 discrete: x(1..24)=input buffer (normalised GHI), x(25)=hold_counter, x(26)=last_prediction |
| Architecture | 5-input NARX (4 past GHI + 1 feedback), 4 hidden tanh units, 1 linear output |
| Weight matrix W1 | 4×5, hand-fitted coefficients |
| Weight vector W2 | 1×5, output layer |
| UART hold | Stochastic hold of 3–10 steps (300 ms – 1 s) modelled via abs(sin(t×17.3)) |
| Output | Predicted GHI (W m⁻²), clamped ≥ 0 |

## 4.3  ha_pv_v3.m — PV Panel Model (IEC 61215)

Models a 50 Wp mono-Si PV panel using an empirical power-law I-V curve with NOCT-based thermal derating and 3% soiling factor. The exponent k is analytically derived so that I(Vmp) = Imp exactly.

| Parameter | Value / Description |
| --- | --- |
| States | 0 (pure feedthrough, flag 3 only) |
| Inputs | 3: G (irradiance), Vref (operating voltage), Tj_ext (external junction temp) |
| Outputs | 4: Ipv, Vr, Ppv, Pm (MPP power) |
| STC parameters | Voc₀=21.6 V, Vmp₀=17.8 V, Isc₀=3.0×0.97 A (3% soiling), Imp₀=2.81 A |
| Curve exponent | k=14.2606 (fitted so I(Vmp)=Imp exactly via dP/dV=0 analytical solution) |
| Thermal coefficients | βVoc=−3.4×10⁻³/°C, βVmp=−4.1×10⁻³/°C, αIsc=+0.5×10⁻³/°C |
| Thermal model | Tc = Tamb + (NOCT−20)/800×G + kJ×max(0, Tj_ext−40), NOCT=45°C, Tamb=35°C, kJ=0.04 |
| MPP voltage | Vmp_c = Voc_T × (1/(1+k))^(1/k) — closed-form from dP/dV=0 |

## 4.4  ha_buck_v3.m — Dynamic Buck Converter

Models an IRFB4110-based synchronous buck converter with a full loss model including conduction loss, switching loss (capacitive + gate charge + turn-on transition), and inductor DCR copper loss. Operating duty cycle is computed dynamically each time step.

| Parameter | Value / Description |
| --- | --- |
| States | 0 (pure feedthrough, flag 3 only) |
| Inputs | 2: Vr (PV-side reference voltage), Vb (battery voltage) |
| Outputs | 3: Io (output current), eta_b (efficiency), Tj (junction temperature) |
| MOSFET params | Rds=3.7 mΩ, Coss=83 nF, Qg=10 nC, Vgs=10 V, tr=20 ns, fsw=50 kHz |
| Inductor | L_DCR=50 mΩ copper loss resistance |
| Diode | Vf=0.45 V forward drop |
| Thermal | Rth=18.55 °C/W, Tamb=40°C → Tj=Tamb+Ploss×Rth |
| Duty cycle | D=clamp(Vb/Vin, 0.01, 0.99) — dynamic per step |

## 4.5  ha_battery_v3.m — SLA Battery Model

Models a 12 V/7 Ah sealed lead-acid battery using the Shepherd OCV equation, Peukert charge integration, and internal resistance voltage drop. Warm-start initial condition avoids a voltage spike at t=0.

| Parameter | Value / Description |
| --- | --- |
| States | 2 discrete: x(1)=Q_Ah (charge), x(2)=Vt_prev (terminal voltage) |
| Inputs | 1: Ic (charge current) |
| Outputs | 2: Vt (terminal voltage), SoC (state of charge 0–1) |
| OCV model | Voc = 11.84 + 1.98×SoC − 0.28×SoC² (Shepherd equation) |
| Peukert | np=1.25, Irate=1.0 A — effective charge dQ=(Ic/Irate)^(np−1)×Ic×dt/3600 |
| Internal resistance | Rint=0.05 Ω → Vt=Voc+Rint×Ic |
| Initial SoC | SoC₀=0.30 (30% at midnight); Vt₀ pre-computed at Ic=0 to avoid spike |

## 4.6  ha_artemis_v3.m — VS-P&O MPPT Controller

Implements the Artemis variable-step perturb-and-observe (VS-P&O) MPPT algorithm enhanced with an LSTM-blend correction step and ADC noise modelling. The controller dynamically scales the perturbation step based on |dP/dV| and applies a LSTM-driven voltage correction when the irradiance prediction diverges significantly from the measured value.

| Parameter | Value / Description |
| --- | --- |
| States | 4 discrete: x(1)=Vref, x(2)=Pprev, x(3)=Vprev, x(4)=cooldown_counter |
| Inputs | 6: Ip, Vp, Gp (predicted), Gn (measured), Vb, Pm |
| Outputs | 3: Vref, eta (MPPT efficiency), cs (charge state) |
| P&O step law | dl = clamp(0.008×¦dP/dV¦, 0.05 V, 0.60 V) — variable step |
| ADC noise | σ_I=2 mA, σ_V=10 mV — added to current and voltage measurements |
| LSTM blend threshold | rel_dev = ¦Gp−Gn¦/max(Gp,Gn) > 0.15 triggers blend |
| Blend weight α | 0.45×exp(−1.5×rel_dev) for Gp>Gn; 0.08×exp(−1.0×rel_dev) for Gn>Gp |
| Cooldown | 20 steps (2 s) after each blend event to prevent oscillation |
| Vref clamp | 0.60×Voc_nom to 0.98×Voc_nom (12.96 V to 21.17 V) |
| Charge states | cs=1: bulk (Vt<14.70V), cs=2: absorption (14.70–14.76V), cs=3: float (>14.76V) |

# 5. Simulink Model — HA_MPPT_v3

## 5.1  Solver Configuration

| Parameter | Value |
| --- | --- |
| Solver type | FixedStepDiscrete |
| Fixed step size | 0.1 s |
| Simulation duration | 86400 s (1 full day) |
| Signal logging | Off (manual To Workspace blocks used) |

## 5.2  Block Topology

The model wires five subsystems together at the top level. Two Unit Delay blocks break algebraic loops:

| Block | Subsystem Contents | Algebraic Loop Fix |
| --- | --- | --- |
| Irr | ha_irr_v3 S-function | — |
| LSTM | ha_lstm_v3 S-function | — |
| PVPanel | Mux → ha_pv_v3 → Demux (3→4) | VrefDly: IC=17.8V, Ts=0.1 |
| ArtemisMPPT | Mux → ha_artemis_v3 → Demux (6→3) | — |
| BuckConv | Mux → ha_buck_v3 → Demux (2→3) | TjDly: IC=40°C, Ts=0.1 |
| Battery | ha_battery_v3 → Demux (1→2) | — |

## 5.3  Signal Routing

Key top-level signal connections:

- Irr/1 → PVPanel/1 (G), LSTM/1 (G), ArtemisMPPT/4 (Gn), log_ghi

- VrefDly/1 → PVPanel/2 (Vref); ArtemisMPPT/1 → VrefDly/1 and BuckConv/1

- TjDly/1 → PVPanel/3 (Tj_ext); BuckConv/3 (Tj) → TjDly/1

- PVPanel/1 → ArtemisMPPT/1 (Ip); PVPanel/2 → ArtemisMPPT/2 (Vp)

- LSTM/1 → ArtemisMPPT/3 (Gp predicted); PVPanel/4 → ArtemisMPPT/6 (Pm)

- Battery/1 (Vt) → ArtemisMPPT/5 (Vb) and BuckConv/2 (Vb)

- BuckConv/1 (Io) → Battery/1; BuckConv/2 (eta) → TrmBeta (terminated)

- ArtemisMPPT/3 (cs) → TrmSt (terminated)

## 5.4  Workspace Loggers

| Variable | Signal Source | Description |
| --- | --- | --- |
| log_ghi | Irr/1 | Global horizontal irradiance (W m⁻²) |
| log_ppv | PVPanel/3 | PV output power (W) |
| log_pmpp | PVPanel/4 | True MPP power (W) |
| log_eta | ArtemisMPPT/2 | Instantaneous MPPT efficiency (0–1) |
| log_soc | Battery/2 | Battery state of charge (0–1) |
| log_vbat | Battery/1 | Battery terminal voltage (V) |
| log_tj | BuckConv/3 | MOSFET junction temperature (°C) |

# 6. Physics Implementation Notes

| Tag | Name | Description |
| --- | --- | --- |
| [F1] | LSTM lag | Stochastic UART hold (3–10 steps) delays irradiance prediction; models real STM32↔ESP32 UART latency at 100 ms polling interval. |
| [F2] | NOCT thermal | Cell temperature uses NOCT correction plus a thermal cross-coupling term from MOSFET Tj when converter runs hot (kJ=0.04). |
| [F3] | Shepherd battery | OCV polynomial fit to SLA discharge curves; Peukert exponent np=1.25 models rate-dependent capacity loss. |
| [F4] | Dynamic buck | Duty cycle D=Vb/Vin computed every 0.1 s; not fixed. Loss model separates conduction, switching, and DCR contributions. |
| [F5] | 0.1 s step / 30 s cloud | Simulation step matches UART polling rate; Markov cloud transitions every 15 s, within the LSTM 30-s prediction horizon. |

# 7. Dependencies

| Requirement | Details |
| --- | --- |
| MATLAB | R2018b or later (simsizes API, add_block/add_line Simulink API) |
| Simulink | Required for new_system, add_block, save_system, sim() |
| Working directory | Must be writable — all 6 S-function .m files are created in pwd |
| No Toolboxes required | No DSP, Control System, or other toolboxes needed beyond base Simulink |