Artemis_Helios_Figures Documentation

Technical Documentation — Helios-Artemis IEEE Publication Figures

Hussain Touhid Siddiquee  |  Dept. EEE, Leading University Sylhet  |  Helios-Artemis Research Project

# 1. Purpose

gen_figures_hires.py generates all nine publication-quality figures required for the Helios-Artemis MPPT paper submission. Every figure targets the IEEE two-column format at 300 DPI using Times New Roman typography. The script is fully self-contained — it does not depend on simulation output files from MATLAB. Instead it re-implements the core physics models (irradiance, PV, P&O) and uses fixed random seeds so figures are exactly reproducible across machines and runs.

# 2. Quick-Start

Run from the project root directory:

pip install numpy matplotlib scipy

python gen_figures_hires.py

Output is written to a figures/ subdirectory created automatically adjacent to the script. Each figure is saved as a PNG at 300 DPI.

NOTE  matplotlib backend is set to 'Agg' at the top of the file. Do not remove this line — without it the script will fail on headless servers (CI, SSH sessions, WSL without display).

# 3. Global Configuration

## 3.1  Canvas & Font Settings

| Parameter | Value / Notes |
| --- | --- |
| Column widths (C1, C2) | 3.5 in (single column), 7.16 in (full two-column) — IEEE standard |
| Font family | serif → ['Times New Roman', 'DejaVu Serif'] fallback |
| Base font size | 8 pt (axis labels), 8.5 pt (titles), 7 pt (ticks/legends) |
| Line width | 1.0 pt (data lines), 0.6 pt (axis spines), 0.35 pt (grid) |
| DPI | 300 for both display and savefig |
| Padding | savefig.bbox='tight', pad_inches=0.04 — minimal whitespace |

## 3.2  Colour Palette

| Variable | Hex | Usage |
| --- | --- | --- |
| B | #1565C0 | Primary blue — irradiance, measured data, primary bars |
| R | #C62828 | Red — baselines, envelope curves, alarm/peak values |
| G | #2E7D32 | Green — Helios-Artemis results, battery/SoC |
| O | #E65100 | Orange — warnings, cost comparisons, literature bands |
| P | #6A1B9A | Purple — LSTM/prediction, step-size law, η traces |
| GR | #546E7A | Grey — secondary text, plain P&O baseline |

## 3.3  Random Seeds

| Seed | Scope | Purpose |
| --- | --- | --- |
| np.random.seed(23) | Global (top of file) | Controls all stochastic content in fig2, fig5, fig7 |
| np.random.seed(7) | Local in fig4() | Generates LSTM test-set irradiance independently of global seed |

WARN  The fig4 suptitle hard-codes R²=0.917, MAE=50.7 W m⁻² (paper-reported values). The live-computed metrics from seed=7 will differ slightly. This is a cosmetic mismatch only — the subplot (a) title shows the live values correctly.

# 4. Shared Physics Helper: pv_model()

A single PV model function is shared by fig3, fig5, and fig7 to ensure consistency across all figures that show PV output. It implements the same IEC 61215 power-law model used in ha_pv_v3.m.

| Argument / Return | Description |
| --- | --- |
| G | Irradiance (W m⁻²) |
| Vr | Reference/operating voltage (V) |
| Isc0=2.91 A | STC short-circuit current (post-soiling: 3.0 × 0.97) |
| Voc0=21.6 V | STC open-circuit voltage |
| k=14.2606 | I-V curve exponent — analytically fitted so I(Vmp)=Imp exactly |
| bV=−3.4×10⁻³ /°C | Voc temperature coefficient |
| aI=+0.5×10⁻³ /°C | Isc temperature coefficient |
| NOCT=45°C, Tamb=35°C | Thermal derating parameters |
| Returns: (Ppv, Pm, Vrc) | PV output power (W), true MPP power (W), clamped operating voltage (V) |

# 5. Figure Reference

## Fig. 1 — System Architecture  (fig1_architecture.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 3.5 in, full two-column width |
| Technique | FancyBboxPatch boxes + annotate arrows — pure matplotlib, no images |
| Blocks | 7 boxes: PV Panel (yellow), Buck Conv (pink), Battery (green), Load, ARTEMIS (purple), HELIOS (blue), Sensors (orange) |
| Key annotation | Bidirectional UART 100 ms arrow (purple) between ARTEMIS and HELIOS with Vref_pred label |
| Hardware annotated | STM32F103 (ARTEMIS), ESP32-S3 (HELIOS), INA219, TSL2591, OV2640, IRFZ44N, TC4420 |
| Output file | fig1_architecture.png |

## Fig. 2 — Irradiance Model  (fig2_irradiance.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 3.5 in, 2 rows × 1 col |
| Subplot (a) | Full 24-hour GHI fill + clear-sky dashed envelope (aerosol ×0.93, peak 744 W m⁻²) |
| Subplot (b) | Zoom 09:30–11:30 h showing OU flicker (τ=1 s, σ=25%) |
| Physics parameters | Markov transition matrix 3×3, cloud fractions [0.15, 0.55, 0.90], Sylhet sunrise 05:30 / sunset 18:30 |
| Random seed | 23 (global) |
| Output file | fig2_irradiance.png |

## Fig. 3 — I-V and P-V Curves  (fig3_iv_curves.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 2.9 in, 1 row × 2 col |
| Subplot (a) | I-V curves for G = 200, 400, 600, 800, 1000 W m⁻². Dots mark true MPP per curve. |
| Subplot (b) | P-V curves for same 5 irradiance levels. Legend lists peak powers. |
| Model used | pv_model() — NOCT thermal derating, k=14.2606 analytical fit, Tamb=35°C |
| Title note | Isc₀=2.91 A reflects 3% soiling factor applied at STC |
| Output file | fig3_iv_curves.png |

## Fig. 4 — LSTM Predictor Performance  (fig4_lstm.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 3.8 in, 2 rows × 2 col |
| Subplot (a) | 60-minute measured vs predicted GHI timeseries (N=600 steps at 0.1 s) |
| Subplot (b) | Scatter: measured vs predicted on independent Year-2 test set. Shows R², MAE, RMSE text box. |
| Subplot (c) | Residual histogram with Gaussian overlay |
| Subplot (d) | α blend-weight sensitivity: MPPT efficiency vs α over [0.05, 0.75]. Optimal α=0.35 marked. |
| LSTM model | Synthetic NARX approximation: pred[i] = 0.80×act[i−1] + 0.20×act[i−4] + noise(σ=18) |
| Random seed | 7 (local, independent of global) |
| Suptitle discrepancy | Hard-coded R²=0.917 / MAE=50.7 in suptitle; live subplot (a) title shows computed values from seed=7 |
| Output file | fig4_lstm.png |

## Fig. 5 — Full-Day Simulation  (fig5_simulation.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 5.8 in, 6-panel stacked (GridSpec, hspace=0.06) |
| Panel 1: GHI | Filled irradiance trace; peak 744 W m⁻² annotated |
| Panel 2: Power | Ppv (green) vs Pmpp dashed (red); legend top-left |
| Panel 3: η_MPPT | Efficiency trace (purple); NaN below G=80 W m⁻²; mean efficiency annotated |
| Panel 4: SoC | State of charge 0–100%; starts at 45%, ends at 100% |
| Panel 5: Vbat | Battery terminal voltage 12.41–13.61 V range annotated |
| Panel 6: Tj | MOSFET junction temperature (°C); peak annotated |
| Simulation model | Inline Python replication of all 5 S-functions at 0.1 s timestep; 864,000 iterations |
| Efficiency formula | sum(Ppv where G>80) / sum(Pmpp where G>80) × 100% |
| Random seed | 23 (global) |
| Output file | fig5_simulation.png |

## Fig. 6 — Method Comparison & Monte Carlo  (fig6_comparison.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 3.0 in, 1 row × 2 col |
| Subplot (a) | Grouped bar chart: 3 methods × 2 conditions (Monsoon July / Annual equiv.). Orange band = literature 90–95% range. |
| Methods compared | Plain P&O (70.7% / 85.8%), VS-P&O without LSTM (85.2% / 88.1%), Helios-Artemis (94.0% / 94.0%) |
| Subplot (b) | 30-day Monte Carlo histogram for Helios-Artemis. Fixed array of 30 values (not random). Gaussian fit overlay. |
| Monte Carlo stats | μ=94.0%, σ≈0.44%, 95% CI annotated. Values hard-coded (not drawn from simulation). |
| Output file | fig6_comparison.png |

## Fig. 7 — VS-P&O Convergence  (fig7_po_convergence.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 2.7 in, 1 row × 3 col |
| Subplot (a) | P-V curve at G=500 W m⁻² with VS-P&O trajectory (70 steps from cold start V=8V). True Vmp dashed line. |
| Subplot (b) | Step-size law: dl = clip(0.008×¦dP/dV¦, 0.05, 0.60). Shows dl_min and dl_max dashed lines. |
| Subplot (c) | Transient convergence: MPPT efficiency vs step count. ~18 steps to reach 94% steady-state. |
| ADC noise model | σ_I=2 mA, σ_V=10 mV applied per step — same values as ha_artemis_v3.m |
| Output file | fig7_po_convergence.png |

## Fig. 8 — Bill of Materials & Cost  (fig8_cost.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 2.9 in, 1 row × 2 col |
| Subplot (a) | Pie chart of BOM cost breakdown across 7 components. Total printed in BDT and USD (÷110). |
| BOM line items | ESP32-S3 (380), STM32F103 (120), INA219 (80), TSL2591 (120), Buck stage (350), PCB+Housing (280), Misc (170) — all BDT Dhaka Q1 2026 |
| Subplot (b) | Bar chart: Helios-Artemis vs 3 commercial alternatives (PWM ctrl, basic MPPT, IDCOL MPPT). Cost in BDT. |
| Saving annotation | ↓xx% vs IDCOL text label auto-computed from BOM total vs 13,500 BDT |
| Exchange rate | 1 USD = 110 BDT (Q1 2026, Dhaka retail) |
| Output file | fig8_cost.png |

## Fig. 9 — Field Validation  (fig9_validation.png)

| Property | Details |
| --- | --- |
| Canvas | 7.16 × 2.9 in, 1 row × 2 col |
| Subplot (a) | 12-month line chart: Hossain et al. field data (red), Helios-Artemis simulation (blue), Plain P&O (grey). ±2% field uncertainty band in red. |
| Subplot (b) | Grouped bar chart for monsoon season May–Oct: Plain P&O vs Hossain et al. vs Helios-Artemis. |
| Reference data | Hossain et al. IDCOL SHS field values: [82,83,85,86,84,80,79,81,83,85,84,82]% |
| Simulation values | Helios-Artemis monthly: [89,90,92,93,91,88,94,91,92,93,91,90]%. July peak = 94% matches Monte Carlo. |
| Validation caveat | Partial validation only — field deployment data not yet available. See paper limitations section. |
| Output file | fig9_validation.png |

# 6. Output File Summary

| # | Filename | Canvas (in) | Subplots | IEEE Use |
| --- | --- | --- | --- | --- |
| 1 | fig1_architecture.png | 7.16 × 3.5 | 1 | Section II |
| 2 | fig2_irradiance.png | 7.16 × 3.5 | 2 (stacked) | Section III |
| 3 | fig3_iv_curves.png | 7.16 × 2.9 | 2 (side) | Section III |
| 4 | fig4_lstm.png | 7.16 × 3.8 | 2×2 grid | Section IV |
| 5 | fig5_simulation.png | 7.16 × 5.8 | 6 (stacked) | Section V |
| 6 | fig6_comparison.png | 7.16 × 3.0 | 2 (side) | Section V |
| 7 | fig7_po_convergence.png | 7.16 × 2.7 | 3 (side) | Section IV |
| 8 | fig8_cost.png | 7.16 × 2.9 | 2 (side) | Section VI |
| 9 | fig9_validation.png | 7.16 × 2.9 | 2 (side) | Section V |

# 7. Dependencies

| Package | Usage | Install |
| --- | --- | --- |
| numpy | Array operations, random number generation, statistics | pip install numpy |
| matplotlib | All figure rendering, FancyBboxPatch, GridSpec, Agg backend | pip install matplotlib |
| scipy.stats.norm | Gaussian overlay on histograms (fig4, fig6) | pip install scipy |
| os (stdlib) | Output directory creation, file path handling | — (built-in) |

# 8. Reproducibility Guarantee

Exact figure reproduction is guaranteed provided:

- numpy version is ≥1.17 (random seed API unchanged since 1.17)

- matplotlib version is ≥3.3 (FancyBboxPatch pad parameter stable)

- scipy version is ≥1.4

- The global seed np.random.seed(23) and local seed np.random.seed(7) in fig4() are not modified

- The script is run as __main__ (not imported as a module, which would skip the seed calls in the module body)

NOTE  If figures are run individually (e.g. calling fig5() in isolation) without first running fig1()–fig4(), the global seed will be at a different RNG state. Always run the full script via python gen_figures_hires.py for exact paper figures.

Helios-Artemis MPPT Research Project  —  gen_figures_hires.py Documentation