Figure 1: System Architecture

Helios-Artemis — Figure Explanation (MDPI Energies Format)

In simple terms: This figure is a block diagram showing how the two microcontrollers (Artemis and Helios) connect to the solar panel, battery, sensors, and each other to track maximum power from the sun.

## 1. What This Figure Shows

Figure 1 presents the overall system architecture of Helios-Artemis — a dual-microcontroller solar charge controller that predicts sunlight changes to harvest more power. The diagram shows how all hardware components connect and communicate. The system uses two chips: ARTEMIS (for fast real-time control) and HELIOS (for smart prediction), connected by a serial link.

## 2. Panel-by-Panel Guide

This figure has a single diagram (no sub-panels) with the following main sections:

Power Flow (top row): A 50-watt monocrystalline silicon solar panel feeds power through a buck converter (a circuit that steps voltage down efficiently) to charge a 12V, 7Ah sealed lead-acid battery. The battery powers a DC load (like lights or a fan). The buck converter switches at 50,000 times per second (50 kHz) for smooth control.

ARTEMIS — STM32F103 (bottom center): This is the 'muscle' microcontroller. It runs the Variable-Step Perturb & Observe (VS-P&O) MPPT algorithm — a smart version of the classic 'trial-and-error' method for finding the solar panel's best operating voltage. It also handles constant-current/constant-voltage (CC/CV) battery charging to protect the battery. An INA219 sensor measures current and voltage.

HELIOS — ESP32-S3 (bottom right): This is the 'brain' microcontroller. It runs two LSTM (Long Short-Term Memory) neural networks — one predicts sunlight (irradiance), the other adjusts the gain. It can retrain itself using TensorFlow.js. It logs data to an SD card and hosts a web dashboard for monitoring — all without needing cloud internet (zero cloud dependency).

Sensors (bottom left): A TSL2591 light sensor measures ambient light, an OV2640 camera captures sky images (for cloud analysis), and an INA219 measures electrical parameters. Data is stored on an SD card.

Communication: ARTEMIS and HELIOS talk to each other every 100 milliseconds over a UART serial link. HELIOS sends the predicted optimal voltage (Vref_pred) to ARTEMIS, which uses it as a starting point for its fine-tuning.

## 3. Key Numbers Made Simple

• 50 watts: Size of the solar panel (enough to power a few LED lights and charge a phone)
• 12V, 7Ah: Battery size (stores about 84 watt-hours — enough for 4-5 hours of lighting)
• 50 kHz: How fast the buck converter switches (50,000 times per second — too fast to hear)
• 100 ms: How often the two chips exchange data (10 times per second)
• 2 microcontrollers: ARTEMIS for fast control, HELIOS for smart prediction

## 4. Why It Matters

This architecture separates real-time control (which must happen every few milliseconds) from prediction (which needs more computing power but can run slightly slower). This dual-MCU design costs about $14 total but delivers over 94% tracking efficiency — matching expensive commercial chargers that cost 5-9x more. The zero-cloud design means it works without internet, making it ideal for rural Bangladesh.